Foshan High Dream Intellectualized Meter Co., Ltd. Address: No.3 Longzhan Road, Huanglong, Beijiao, Shunde, Foshan, 528311, Guangdong China.

Tel: 0757-22393066	  Fax: 0757-22393080

Email: highdream@126.com 	Official Website: www.hcznyb.com

High Dream Intellectualized Meter

Guangdong, China

From Every Stream, To Every Dream

Intelligent Meters: Professional and Focused, Embracing All Rivers to Lead Innovation

Contents

Contents

Group Introduction

Product Introduction

IoT Water Meter

Remote Transmission Water Meter

Prepaid Water Meter

Ultrasonic Smart Water Meter

Mechanical Water Meter

Smart Water Utilities

Supporting Equipment

Market Services

Group Introduction

Guangdong High Dream Intellectualized Machinery Co., Ltd.

Stock Code: 300720

120+

Countries around the world

50+

Domestic and International  patents

70000+

M2 plant area

450+

Well-trained employees

Development Goals

Ntegrity in business operations, provision of high-quality services, and continuous research and innovation. While comprehensively enhancing the industrial intelligent services of products, we will rely on the international management model, innovative business philosophy, and strong operational strength to strive to build into an international professional metering system.

Foshan High Dream Intellectualized Meter Co., Ltd.

Foshan High Dream Intellectualized Meter Co., Ltd. is a holding enterprise of Guangdong High Dream Intellectualized Machinery Co., Ltd. (Stock Code: 300720). It is a high-tech enterprise specializing in the research of industrial chains and standards related to the water industry, as well as the development, manufacturing, sales and service of its products. The main businesses include: LoT water meter , wireless remote transmission water meter , wired remote transmission water meter , prepaid water meter , ultrasonic intelligent water meter , mechanical water meter series, etc.; R&D, production, sales, consultation and technical services of intelligent meter reading terminals, computer software and system integration, etc.

The business covers the whole country and covering multiple aspects such as basic research and development, product development, marketing and sales, technical support, and education and training.Our services meet the inherent needs of several water utility companies in water - saving and safe water supply, and have been recognized.

Cooperation • Win-Win • Innovation • Development

Certificates

Honor and Qualification

Patent

Certification

Honor and Qualification

Patent

Certification

Remote Meter Reading System

Application Layer

Communication + Acquisition Layer

Mobile Terminal

Front-end Machine

Smart Water Cloud Platform

Customer Meter Reading Platform

UDP Direct Connection

Cloud Platform Push

Back-end Server

IoT Platform

NB/4G Base Station

IoT Water Meter

IoT  Valve Water Meter

IoT  Ultrasonic Water Meter

IoT  Bulk  Water Meter

(with Valve/without Valve)

(with Valve/without Valve)

IoT Water Meter Series

NB-IoT Water Meter

This series of smart remote transmission water meters developed based on NB-IoT communication technology. It features low power consumption, long-range communication, strong anti-interference capability, and stable data transmission.

Internal structure: it adopts a combination of ultra-low power consumption pulse sampling method and low-power NB-IoT transmission module.

External structure(A split design): the electronic unit is installed in an independent fully sealed cavity, with a waterproof performance of IP68 rating. It maintains the structure and accuracy of traditional wet-type water meters.

NB-IoT Water Meter (without Valve)

Product Features

| · NB-IoT networking communication · No additional centralized collection equipment required · Each water meter is an independent terminal · Convenient construction and installation, no wiring required · Suitable for situations with dispersed meter installations. | · Communication is mainly affected by the signal of the operator's base station · Meter reading data is actively reported, and control commands are issued remotely · The number of reports and time can be freely set · with Valve water meters can achieve prepayment when combined with the upper system · Supports emergency valve opening function in emergency situations |
| --- | --- |

Technical Parameters

Installation Dimension

| Nominal Diameter (DN), mm |  |  |  |
| --- | --- | --- | --- |
| Permanent Flow Rate (Q3), m3/h |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |
| Metering Accuracy Class | Class 2 |  |  |
| Max. Pressure |  |  |  |
| Pressure Loss |  |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |  |
| Flow Profile Sensitivity |  |  |  |
| Communication Interface | Wireless: NB-IoT Infrared: 9600bps |  |  |
| Operating Frequency Band |  |  |  |
| Installation |  |  |  |
| Environmental Class | Class B |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |
| Waterproof Rating | IP 68 |  |  |
| Working Voltage | 3.6V lithium battery, ER26500/ER34615 |  |  |
| Case Material | Ductile Iron/Stainless Steel/Copper |  |  |

DN15~DN25 (with Valve))	DN15~DN25 (without Valve))

| Model | DN (mm) | L (mm) | L1 (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- | --- | --- |

NB-IoT Water Meter (with Valve)

NB-IoT Water Meter (Non-magnetic Sensing)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

IoT Water Meter Series

4G IoT Water Meter

This is a new type of smart water meter that uses a Rotary Type Water Meter as its base.

Internal structure: Equipped with an integrated communication module and SIM card. 4G network enables direct transmission of water meter-collected data (flow rate, temperature, signal strength) to the meter reading system, achieving data collection, remote control, and fault alarm.

External structure(A split design): the electronic unit is installed in an independent fully sealed cavity, with a waterproof performance of IP68 rating. It maintains the structure and accuracy of traditional wet-type water meters.

4G Smart Water Meter (without Valve)

Product Features

·	Mechanical & Electrical Design: Modular structure (mechanical & electronic parts), screw-assembled for easy maintenance.

·	Network Coverage: NB & 4G hybrid, wide coverage for rural, urban, indoor scenarios.

·	Communication: Direct base station connection, supports CT AEP, CM OneNET, UDP protocols.

·	Measurement: Non-magnetic/magnetoresistive sampling, accuracy 1L/10L; The electronic section: equipped with bi-directional metering capability,  synchronized with base meter to avoid disputes.

·	Emergency Valve (Valve-controlled): Opens automatically in emergencies without network signal, maintaining water supply.

·	Multiple modes of Data Reporting: daily, N times/day, every N days, specified dates; N≥2.

·	Time-sharing Reporting: Prevents server congestion from a large number of simultaneous terminal reports.

·	Abnormality Alarm: Alerts for power failure, sensor faults, valve faults.

·	Prepaid Function (Valve-controlled): Enables back-end prepaid with upper system.

Technical Parameters

Installation Dimension

| Nominal Diameter (DN), mm |  |  |  |
| --- | --- | --- | --- |
| Permanent Flow Rate (Q3), m3/h |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |
| Metering Accuracy Class | Class 2 |  |  |
| Max. Pressure |  |  |  |
| Pressure Loss |  |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |  |
| Flow Profile Sensitivity |  |  |  |
| Communication Interface | Wireless: 4G_Cat.1 Infrared: 9600bps |  |  |
| Operating Frequency Band |  |  |  |
| Installation |  |  |  |
| Environmental Class | Class O |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |
| Waterproof Rating | IP 68 |  |  |
| Working Voltage | 3.6V Lithium Battery, ER26500/ER3461 5 |  |  |
| Case Material | Ductile Iron/Stainless Steel/Copper |  |  |

DN15~DN25 (with Valve)	DN15~DN25 (without Valve)

| Model | DN (mm) | L (mm) | L1 (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- | --- | --- |

4G Smart Water Meter (Non-magnetic Sensing)

4G Smart Water Meter (with Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

Direct Drinking Water Meter Series

IoT Direct Drinking Water Meter

This is a Smart water meter with Volumetric type and Rotary Type Water Meter as its base.

Internal structure: Equipped with an integrated communication module and SIM card. 
4G network/NB-IoT enables direct transmission of water meter-collected data (flow rate, temperature, signal strength) to the meter reading system, achieving data collection, remote control, and fault alarm.

Product Features

·	Strong anti-interference: Magnetoresistive sampling, 1L accuracy; The electronic section: equipped with bi-directional metering capability,  synchronized with base meter to avoid disputes.

·	Protection: IP68 rating, dual sealing (structural + potting) for excellent waterproof performance.

·	Mech-Electrical Design: Modular (mechanical & electronic parts), screw-assembled for easy maintenance.

·	Parameter Setting: Configurable via external infrared port and front-end system.

·	Touch Wake-up Function: Activated by touch button or magnetic switch (≥3s), activate the trigger module to report data and infrared communication.

·	Platform Communication: Supports AEP/OneNet transparent transmission, UDP direct connection, and CT/CM/CU thing models; fully compatible with IoT platform NB-IoT meters for seamless switching.

·	Data Retransmission: Auto-retransmits failed data in next cycle; retransmission cycle and count configurable.

·	Reporting Frequency: 4 modes (daily, multiple times/day, every multiple days, specified dates).

·	Time-sharing Reporting: Prevents server congestion from a large number of simultaneous terminal reports.

·	Emergency Valve (Valve-controlled): Opens automatically in emergencies without network signal, maintaining water supply.

·	Overflow Valve Closing: Auto-closes when consumption exceeds set value; reopens manually to avoid waste from leakage or forgotten faucets.

Technical Parameters

Installation Dimension

| Nominal Diameter (DN), mm |  |  |
| --- | --- | --- |
| Permanent Flow Rate (Q3), m3/h |  |  |
| Measurement Range (Q3/Q1) |  |  |
| Metering Accuracy Class | Class 2 |  |
| Max. Pressure |  |  |
| Pressure Loss |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |
| Flow Profile Sensitivity |  |  |
| Communication Interface | Wireless: NB-IoT/4G_Cat.1 Infrared: 9600bps |  |
| Operating Frequency Band |  |  |
| Installation |  |  |
| Environmental Class | Class O |  |
| Electromagnetic Compatibility | Class E1 |  |
| Waterproof Rating | IP 68 |  |
| Working Voltage | 3.6V Lithium Battery, ER26500/ER3461 5 |  |
| Case Material | Stainless Steel |  |

DN15 (with Valve)	DN8 (without Valve)

| Model | DN (mm) | L (mm) | L1 (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- | --- | --- |

Direct Drinking Water Meter

（without Valve）

Prepaid Direct Drinking Water Meter

IoT Water Meter Series

IoT Water Meter (Bulk)

This is a new type of smart water meter that uses a Turbine Type Water Meter as its base.

Internal structure: Equipped with an integrated communication module and SIM card. 4G network/NB-IoT enables direct transmission of water meter-collected data (flow rate, temperature, signal strength) to the meter reading system, achieving data collection, remote control, and fault alarm.

External structure(A split design): The electronic device of woltman meters(DN40~DN200) can be fixed on the base meter or led out and installed on the wall. The whole machine has a waterproof performance of IP68, which maintains the structure and accuracy of traditional wet-type water meters.

Product Features

·	Network Coverage: NB & 4G hybrid, wide coverage for rural, urban, indoor scenarios.

·	Communication: Direct base station connection, supports CT AEP, CM OneNET, UDP protocols.

·	Measurement: Non-magnetic/Hall sampling, accuracy 0.1m³/1m³; The electronic section: equipped with bi-directional metering capability,  synchronized with base meter to avoid disputes.

·	Data Reporting: Multiple modes (daily, N times/day, every N days, specified dates) (N≥2).

·	Time-sharing Reporting: Prevents server congestion from a large number of simultaneous terminal reports.

·	Abnormal Alarm: Alerts for power failure, sensor abnormality, disconnection.

·	Battery & Application (DN40~DN200 Turbine type): Optional multi-section ER34615 battery packs, supporting intensive collection/reporting.

·	Pressure Sensor (Vertical Turbine water meter): Optional installation.

Technical Parameters

Installation Dimension

| Base Meter Type | Rotary Type |  | Turbine Type |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter (DN), mm |  |  |  |  |  |  |  |
| Permanent Flow Rate (Q3), m3/h |  |  |  |  |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |  |  |  |  |
| Max. Pressure |  |  |  |  |  |  |  |
| Pressure Loss |  |  |  |  |  |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |  |  |  |  |  |
| Flow Profile Sensitivity |  |  |  |  |  |  |  |
| Communication Interface | Wireless: NB-IoT/4G_Cat.1 Infrared: 9600bps |  |  |  |  |  |  |
| Operating Frequency Band |  |  |  |  |  |  |  |
| Installation |  |  |  |  |  |  |  |
| Environmental Class | Class O |  |  |  |  |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |  |  |  |  |
| Waterproof Rating | IP 68 |  |  |  |  |  |  |
| Working Voltage | 3.6V lithium battery, ER26500/ER34615 |  |  |  |  |  |  |
| Case Material | Ductile Iron/Stainless Steel/Copper |  |  |  |  |  |  |

(Rotary Type)

Woltman Meter (WPH)

Vertical Woltman Meter (WS)

Rotary Water Meter

(Turbine Type)

| (DN) | L (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- |

| DN (mm) | L (mm) | H (mm) | Flange Outer Diameter D (mm) | Bolt Hole Center Circle Diameter d (mm) | Connecting Bolt and Quantity |
| --- | --- | --- | --- | --- | --- |
|  |  |  | G1 1/2 (Thread Connection d) |  | G2B (Thread Connection D) |

Remote Transmission Water Meter Series

Wireless Remote Water Meter

This is a remote water meter that uses Rotary Type Water Meter as its base.

Adopting LoRa communication technology, transmits data such as flow rate, battery voltage, and signal strength collected by the water meter to the meter reading system through a concentrator, achieving data collection, remote control, and fault alarm.

External structure(A split design): the electronic unit is installed in an independent fully sealed cavity, with a waterproof performance of IP68 rating. It maintains the structure and accuracy of traditional wet-type water meters.

Wireless Remote Transmission Water Meter (without Valve)

Product Features

·	Double protection of structural sealing and potting glue sealing, with IP68

·	Mechanical and electrical separation structure design for convenient later maintenance

·	LoRa wireless communication technology, point-to-point networking mode, supports multi-level relay mode for meter end and repeater networking

·	Fault alarm: supports low battery alarm/magnetic interference alarm/valve abnormality alarm

·	The system can perform real-time meter reading and valve control without operation lag issues

·	Open straight-line communication distance of 1km, which can be extended to about 5km through relay equipment

Technical Parameters

Installation Dimension

| Nominal Diameter (DN), mm |  |  |  |
| --- | --- | --- | --- |
| Permanent Flow Rate (Q3), m3/h |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |
| Metering Accuracy Class | Class 2 |  |  |
| Max. Pressure |  |  |  |
| Pressure Loss |  |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |  |
| Flow Profile Sensitivity |  |  |  |
| Communication Method | Wireless: LoRa |  |  |
| Operating Frequency Band |  |  |  |
| Installation |  |  |  |
| Environmental Class | Class B |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |
| Waterproof Rating | IP 68 |  |  |
| Working Voltage | 3.6V lithium battery, ER26500/ER34615 |  |  |
| Case Material | Ductile Iron/Stainless Steel/Copper |  |  |

DN15~DN25 (with Valve)	DN15~DN25 (without Valve)

| Model | DN (mm) | L (mm) | L1 (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- | --- | --- |

Wireless Remote Transmission Water Meter (with Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

(with Valve/without Valve)

Remote Transmission Water Meter Series

Wired Remote Water Meter

Photoelectric Direct-reading Wired Remote Water Meter uses electronic remote transmission technology to directly read character wheel data.

Compared with conventional pulse water meters, it can reduce reading errors to zero, making it an electronic remote transmission water meters with zero error in electromechanical conversion in automatic meter reading systems.

Moreover, it adopts a low-power design, requiring no power supply normally and only needing power supply during meter reading.

Product Features

| •M-bus or RS485 communication •Simple wiring, low cost, high meter reading success rate •Zero electromechanical error: Optical character wheel recognition technology directly reads the water meter character wheel reading | •Meter reading is not affected by time, environment, or back-flow •Bus current expansion power supply with strong load capacity •Meter reading data supports active reporting and remote issuance of meter reading commands |
| --- | --- |

Interface Parameters

| Parameter Interface | Baud Rate | Protocol | Address Code | Dial Indication | Voltage |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Last 8 Digits | Direct Current |
| Bus |  |  |  | Last 12 Digits |  |
|  |  |  |  | Last 12 Digits |  |
| Bus |  |  |  | Last 2 Digits |  |

Technical Parameters

Installation Dimension

| Base Meter Type | Rotary Type |  |  |  | Turbine Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal diameter (DN), mm |  |  |  |  |  |  |  |  |  |  |
| Permanent Flow Rate (Q3), m3/h |  |  |  |  |  |  |  |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |  |  |  |  |  |  |  |
| Max. Pressure |  |  |  |  |  |  |  |  |  |  |
| Pressure Loss |  |  |  |  |  |  |  |  |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |  |  |  |  |  |  |  |  |
| Flow Profile Sensitivity | U10/D5 |  |  |  |  |  |  |  |  |  |
| Communication Method | Wired: M-BUS/RS485 |  |  |  |  |  |  |  |  |  |
| Communication Protocol | CJ/T188, DL/T645, Modbus (Optional) |  |  |  |  |  |  |  |  |  |
| Installation | H |  |  |  |  |  |  |  |  |  |
| Environmental Class | Class B |  |  |  |  |  |  |  |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |  |  |  |  |  |  |  |
| Waterproof Rating | IP 68 |  |  |  |  |  |  |  |  |  |
| Working Voltage | RS485 interface: 7V-12V M-BUS interface: 24V~36V |  |  |  |  |  |  |  |  |  |
| Case Material | Ductile Iron/Copper |  |  |  |  |  |  |  |  |  |

DN15~DN40 (Rotary Type)

Photoelectric Direct-reading Wired

Remote Water Meter (Small Diameter)

Photoelectric Direct-reading

Wired Remote Water Meter

(Woltman)

Photoelectric Direct-reading

Wired Remote Water Meter

(Vertical turbine)

DN40~DN200 (Turbine Type)

| (DN) | L (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- |

| DN (mm) | L (mm) | H (mm) | Flange Outer Diameter D (mm) | Bolt Hole Center Circle Diameter d (mm) | Connecting Bolt and Quantity |
| --- | --- | --- | --- | --- | --- |
|  |  |  | G1 1/2 (Thread Connection d) |  | G2B (Thread Connection D) |

Note: (1) Communication distance: 300m; (2) Minimum data storage time: unlimited; (3) Communication protocol in accordance with the communication specification: CJ/T188-2004 Technical requirements of utility meters data transmission or as agreed by both parties; (4) No power supply normally; during data collection, 485 bus (0.6~0.8) mA, M-BUS bus (1.0~1.3) mA.

Prepaid Water Meter System

Server

Data Acquisition Layer

Perception Layer

Billing System Platform

Server

WIFI/4G/Ethernet

Online

Water Sales

Self-service Payment Terminal

IC Card

Bluetooth 4.0

IC Card Water Meter

Bluetooth Water Meter

IC Card

IC Card

IC Card Water Meter

IC Card Water Meter

Prepaid Water Meter Series

IC Card Smart Water Meter (Bluetooth)

IC Card Cold type Water Meter /IC Card Hot type Water Meter uses a Rotary Wet type Water Meter as its base.

Adopts the electromechanical conversion method of pulse magnetoresistive counting function, completes data transmission via IC card and Bluetooth, and is suitable for occasions where users have the requirement of mobile phone recharge for prepayment before water usage.

Product Features

| · Can connect with mobile phone Bluetooth to realize meter setting, meter reading, and recharging · Remote account opening, allowing users to realize non-card operation throughout the process · Magnetic-sensitive one-way metering, which can prevent errors caused by jitter and self-rotation | · Valve Self-Maintenance, regular valve rotation removes scale, ensuring flexibility. · Flexible setting of recharge limit to prevent users from hoarding water |
| --- | --- |

Flow Parameters

| DN (mm) | Measurement Range Q3/Q1 Q2/Q1 | Overload Flow Rate (Q4), m3/h | Permanent Flow Rate (Q3), m3/h | Transitional Flow Rate (Q2), m3/h | Minimum Flow Rate (Q1), m3/h |
| --- | --- | --- | --- | --- | --- |

Technical Parameters

Installation Dimension

| Performance | Detailed Parameters |
| --- | --- |
| Power Supply | Lithium Manganese Battery 3.0 V, 6-year warranty, easy to replace |
| Display Method | Dual display with LCD and mechanical character wheel, LCD screen off in daily use, screen can be activated by card swiping and Bluetooth communication |
| LCD Display Content | Remaining Amount, Cumulative Consumption, Current Period Consumption, Valve Status (Valve Open/Valve Closed), Low Battery Indicator, Magnetic Attack Indicator |
| Data Collection Method | Pulse Magnetic Resistance, 10 L Position Signal |
| Medium Temperature | Cold Water Meter: 0.1℃~30℃; Hot Water Meter: 0.1℃~90℃ |
| Storage Environment | Relative Humidity ≤ 90% |
| Pressure and Pressure Loss | Water Pressure ≤ 1MPa; Pressure Loss ≤ 0.063MPa |
| Level | Electromagnetic Environment: E1, Strong Magnetic Environment Prohibited; Protection: IP68; Installation Environment: Class O |
| Bluetooth | Maximum Communication Distance in open space: 10m |

IC Card Smart Water Meter (Bluetooth)

Horizontal

Bottom Inlet Vertical

Top Inlet Vertical

| Model | DN(mm) | Control Dimension/mm (Copper) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Length L | Width B | Height H | Thread D | Thread d |

| Model | DN (mm) | Control Dimension/mm (Plastic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Length L | Width B | Height H | Thread D | Thread d |

| Model | DN(mm) | Control Dimension/mm (Plastic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Length L | Width B | Height H | Thread D | Thread d |

| Model | Nominal Diameter (mm) | Control Dimension/mm (Plastic) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Length L | Width B | Height H | Thread D | Thread d |

Prepaid Water Meter Series

IC Card Smart Water Meter

IC Card Cold type Water Meter /IC Card Hot type Water Meter uses a Rotary Wet type Water Meter as its base.

Adopts the electromechanical conversion method of pulse magnetoresistive counting function, completes data transmission via IC card, and is suitable for occasions where users have the requirement of mobile phone recharge for prepayment before water usage.

Product Features

·	HD LCD:  shows purchased water volume, remaining volume, cumulative consumption, valve status, battery status, etc.

·	Prepayment: Pay-first usage; auto-closes valve when out of credit, auto-opens after recharge.

·   Mechanical and electrical separation design: convenient for maintenance in the later stage

·	Prompt & Alarm: LCD prompts and valve-closing alarms for faults (battery power-off, under voltage, low water, strong magnetic interference).

·	Valve Self-Maintenance: Regular valve rotation removes scale, ensuring flexibility.

·	Water Price Setting: Supports single and stepped water price configuration at the meter end.

·	IC Card Payment: Enables self-service payment via payment terminals.

·	Bluetooth Function (Bluetooth Meters): Connects to mobile Bluetooth for meter setting, reading, and recharge.

·	Protection: IP68 rating, dual sealing (structural + potting) for excellent waterproof performance.

Technical Parameters

Installation Dimension

| Nominal Diameter (DN), mm |  |  |  |
| --- | --- | --- | --- |
| Permanent Flow Rate (Q3), m3/h |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |
| Metering Accuracy Class | Class 2 |  |  |
| Max. Pressure |  |  |  |
| Pressure Loss |  |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |  |
| Flow Profile Sensitivity |  |  |  |
| Communication Method | IC Card |  |  |
| Installation |  |  |  |
| Environmental Class | Class B |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |
| Waterproof Rating |  |  |  |
| Case Material | Copper |  |  |

| Model | DN (mm) | L (mm) | L1 (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- | --- | --- |

Ultrasonic Water Meter Series

Ultrasonic Water Meter

This is a new type of fully electronic water meter manufactured with industrial-grade electronic components. It detects the time difference caused by the change in speed when ultrasonic beams propagate downstream and upstream in water, analyzes and processes to obtain the water flow velocity and further calculates the water flow rate.

It has excellent small flow detection capability, micro power consumption, low starting flow, high stability, can solve many problems of conventional water meters.

Specifically used for household metering in residential communities, it meets the needs of water supply enterprises for accurate metering and settlement for end users, and has broad market and application prospects.

Product Features

·	High measurement accuracy:Fluid stratified dual-channel/quad-channel measurement

·	No mechanical rotating parts, no mechanical wear, good flow performance, small pressure loss, suitable for various water qualities

·	Easy installation, with options for horizontal, vertical, and inclined installation

·	Good anti-freezing performance

·	Multiple transmission methods, supporting wired RS485 and wireless NB/4G transmission

·	Double protection of structural sealing and potting glue sealing, with IP68

Technical Parameters

Flow Parameters

| Nominal diameter (DN), mm |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Permanent Flow Rate (Q3), m3/h |  |  |  |  |  |  |  |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |  |  |  |  |  |  |  |
| Max. Pressure |  |  |  |  |  |  |  |  |  |  |
| Pressure Loss |  |  |  |  |  |  |  |  |  |  |
| Working Temperature | Cold Water Meter (T30): 0.1℃~30℃; Hot Water Meter (T90): 0.1℃~90℃ |  |  |  |  |  |  |  |  |  |
| Flow Profile Sensitivity | U5/D3 |  |  |  |  |  |  |  |  |  |
| Communication Method | Wireless: NB-IoT/4G_Cat.1; Wired: RS485; Infrared: 9600bps |  |  |  |  |  |  |  |  |  |
| Communication Protocol | Wireless: internal protocol, customizable development; RS485 interface: CJ/T188, DL/T645, Modbus (optional) |  |  |  |  |  |  |  |  |  |
| Power Supply Method | Built-in battery power supply (default); External power supply: 7V~12V (customized) |  |  |  |  |  |  |  |  |  |
| Installation | H/V |  |  |  |  |  |  |  |  |  |
| Environmental Class | Class O |  |  |  |  |  |  |  |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |  |  |  |  |  |  |  |
| Waterproof Rating | IP 68 |  |  |  |  |  |  |  |  |  |
| Case Material | Stainless Steel/Carbon Steel/Copper |  |  |  |  |  |  |  |  |  |

4G Ultrasonic Water Meter

(Small diameter)

NB-IoT Ultrasonic Water Meter

Wireless Remote Bulk Ultrasonic Water Meter

| DN(mm) | L (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- |

| DN (mm) | Overall Dimensions |  |  |  | Minimum Starting Flow |  |  |  |  | Weight kg | Max. Pressure MPa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Bolt Hole Diameter × Quantity | Sealing Surface | Flange Thickness C |  |  |

Mechanical Water Meter Series

Mechanical Water Meter

Rotary Type Cold Water Meter/Turbine Type Cold Water Meter are specialized instrument suitable for measuring the total volume of water flowing through tap water pipes.

It has characteristics such as high sensitivity, accurate measurement, small head loss, durability, and convenient reading.

Product Features

·	Performance requirements meet the GB/T 778.1~3-2018 standard

·	Featuring compact structure, high metering accuracy, small starting flow, reliable operation, etc.

·	Special wear-resistant materials, processed with error trend gradually corrected, to avoid water fee loss

·	Check valve can be installed as needed to prevent water back-flow

·	Can reserved magnetic or non-magnetic signal pointers. Configured with various remote transmission devices according to different requirements.

Technical Parameters

Installation Dimension

| Base Meter Type | Rotary Type |  |  |  | Turbine Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter (DN), mm |  |  |  |  |  |  |  |  |  |  |
| Permanent Flow Rate (Q3), m3/h |  |  |  |  |  |  |  |  |  |  |
| Measurement Range (Q3/Q1) |  |  |  |  |  |  |  |  |  |  |
| Max. Pressure |  |  |  |  |  |  |  |  |  |  |
| Pressure Loss |  |  |  |  |  |  |  |  |  |  |
| Working Temperature | Cold Water Meter (T3O): 0.1℃~30℃ Hot Water Meter (T90): 0.1℃~90℃ |  |  |  |  |  |  |  |  |  |
| Flow Profile Sensitivity |  |  |  |  |  |  |  |  |  |  |
| Installation |  |  |  |  |  |  |  |  |  |  |
| Environmental Class | Class B |  |  |  |  |  |  |  |  |  |
| Electromagnetic Compatibility | Class E1 |  |  |  |  |  |  |  |  |  |
| Case Material | Ductile Iron |  |  |  |  |  |  |  |  |  |

Rotary Water Meter

Vertical Turbine Water Meter

Woltman Meter

| DN (mm) | L (mm) | H (mm) | Flange Outer Diameter D (mm) | Bolt Hole Center Circle Diameter d (mm) | Connecting Bolt and Quantity |
| --- | --- | --- | --- | --- | --- |
|  |  |  | G1 1/2 (Thread Connection d) |  | G2B (Thread Connection D) |

| DN (mm) | L1 (mm) | H (mm) | W (mm) | Connecting Thread d | Connecting Thread D |
| --- | --- | --- | --- | --- | --- |

DN40~200 (Turbine Type)

DN15~40 (Rotary Type)

Project Overview

Integrated Smart Water Solution

We integrate cloud computing, big data, GIS geographic information, and IoT technologies, conduct in-depth water supply data modeling, pipeline network spatial and hydraulic models to create a customized integrated management platform for water utilities.(water production - water supply - user water consumption)

we assist water utilities in operation and maintenance management in a refined, dynamic, and "intelligent" manner.. Thereby ensuring reliable water supply, reducing leakage and narrowing the production and sales difference, enhancing operational control capabilities.

Smart Marketing

Smart Pipe Network

Big Data of Water Management

DMA Partition

Smart Water Plant

Smart Marketing

Smart Pipe Network

Smart Water Plant

iWater APP

Data Acquisition Platform

Data Synchronization Engine

Ethernet

Application Layer

Data Warehouse

Transport Layer

Perception Layer

Layered Structure

Data Acquisition Platform

Open Multi-vendor Compatibility

The platform has considered compatibility with the access of third-party manufacturers' equipment, and does not set any thresholds for third-party manufacturers' access.

Supports Various Devices

For residential: mechanical water meters, wireless/wired remote transmission water meters

For industrial/commercial : NB-IoT water meters, large-diameter water meters for pipe networks, pressure gauges, flow meters and other pipe network equipment

Supports Multiple Communication Methods

Serial Port, Ethernet, 4G, LoRa, NB-IoT

Platform Features

Diversified Meter Reading Strategies

Can be set with daily, weekly, monthly, single or multiple times meter reading strategies, with active upload.

Supports connections for up to one million devices.

Ensures data collection stability at the scale of over one million devices. Features a collection service cluster and dynamic service balancing, enabling horizontal scalability of the cluster service to be virtually limitless.

Third-party Data Interface

Provides diverse formats and interfaces to facilitate data import and export for third-party systems.

Smart Marketing

Fee Collection and Account Settlement

- Supports multi-channel fee collection ,including: counter, bank, Alipay, and WeChat

- 24-hour service

- Supports paper and electronic invoices

- Supports IC card water meters, mechanical water meters, remote transmission water meters, etc.(One-Stop Solution)

Household Registration and Installation Application

- Supports applying for services on mobile devices.

- Full-process tracking and progress reminders

- Supports approval on mobile devices.

- Customizable processes and forms

Meter Reading Management

- Diversified Water Consumption Data Collection Methods

- SMS Payment Reminder

- Remote Valve Switching

- Automatic Valve Closure for Arrears

- Automatic Valve Opening Upon Payment

- Support Water Fee Daily/Monthly Settlement

Statistical Analysis

- Abnormal Water Usage Analysis

- Meter Reading Rate Statistics

- Payment Collection Rate Statistics

- Monthly and Annual Water Usage Situation Analysis

- Revenue Statistical Report

Smart Marketing

By adopting a B/S (Browser/Server) architecture, a cloud-based service model is implemented to rapidly develop a comprehensive marketing management system for small-scale water utilities. This integrated system includes functionalities such as customer profiling, meter reading management, account management, and statistical reporting, all designed to boost the marketing efficiency and operational analysis capabilities of water utilities.

iWater APP

Based on mobile Internet technology as the basic architecture, it integrates relevant businesses of the water company, applies mobile APP technology to change the conventional business model, and enables staff to handle daily business more conveniently and quickly using the APP.

Mechanical Water Meter Reading

Read the mechanical water meter according to the meter reading book and meter reading path, upload to the server.

Supports online/offline meter reading.

Remote Valve Control

Perform real-time valve opening and closing control for remote transmission water meters via mobile phone.

Mobile Office

Initiates work processes and process workflows, and tracks process progress anytime and anywhere.

Business Management

Management personnel can conveniently and quickly view annual and monthly business data.

DMA Partition Metering

Smart Pipe Network

One Map (Pipeline Network GIS)

The pipeline network direction, valves, large meters, flow meters, pressure points, and waterworks are fully digitized, displayed on a visual map, and never lost.

Pipe Burst Analysis

Simulate pipe burst points, analyze the valves that need to be closed, affected pipelines, and affected users, providing a basis for decision-making.

Hydraulic Model

Analysis of pipeline network node pressure and instantaneous flow provides more scientific and effective auxiliary design support for the design and construction of water supply pipelines.

Water Cut-off Area Analysis

Based on spatial operations and valve closure plans, analyze the areas that will have water cut-offs due to accidents, and notify water users in the water cut-off areas via SMS.

Pipeline Network Monitoring

Monitoring of pipeline network flow and pressure, data analysis, flow and pressure curves, providing data support for water supply operation and maintenance management.

The pipe network is divided into regionalized and grid-based management areas. According to the service area and length of the pipe network, the water supply area can be divided into multiple metered areas (DMA). Using modern wireless communication technology, real-time transmission of data such as flow, pressure, and water quality is carried out, so that the operation status of the pipe network in each metering area can be timely and accurately grasped, providing a scientific basis for optimizing the water supply pipe network.

Production and Sales Difference Analysis

Identify real leakage, normal leakage, and management leakage to provide a basis for pipe network optimization and management improvement.

Night Minimum Flow Analysis

The system can set the baseline value of the minimum night flow for partitions, analyze partitions to evaluate night water usage categories, sudden water usage, or new water leakage. And it can issue alarm information according to the set water leakage threshold and evaluate the economy of repairing new leakage.

Big Data of Water Management

Smart Water Plant

Integrated visual large-screen display system of the leadership cockpit and water dispatch center.

Can refresh equipment operation data at various key points in the pipe network and water plants in real time and dynamically; through big data analysis, all key data can be mastered on one screen.

Real-time Monitoring and Alarm for Pump Room Equipment

Filter Tank Water Level, Turbidity, and Equipment Monitoring Alarm

Real-time Monitoring and Alarm of Dosing Equipment

Various Production Data Reports

Supporting Equipment

Water Tank Liquid Level

Liquid Level Meter

Factory Water Supply

Commercial Water Supply

Residential Area Water Supply

School Water Supply

Adjustable Valve-Size

Smart Electric Meter

HCG-L/M Type Data Concentrator

Water Quality Analyzer

Pressure Analyzer

Flow Meter

Server	 Monitoring Display

Mobile APP	Client	Printer

HCG-L/M

Data Concentrator

Pressure Analyzer

Flow Meter

Water Quality Analyzer

Product Features

Data Collection and Processing Function

Can perform real-time polling or automatically collect relevant data information of remote transmission meters according to the set reading interval;

Timed Automatic Collection

The concentrator automatically collects data from remote transmission meters according to the meter reading plan set by the master station;

Setting Function

Can set automatic meter reading cycle, reading interval, reading interval, etc., and measures to prevent unauthorized personnel from operating;

Expansion Function

Can receive commands from the control center or freeze the cumulative amount of water, gas, and heat meters at a specified real-time time according to the set freezing time;

- NB-IoT network is applicable in a wide range of regions;

- LCD real-time display;

- Antenna types available: external and integrated; external antenna can be selected for areas with weak signals;

- Built-in low-power circuit and 28000mAh large-capacity lithium battery, can work continuously for more than 6 years in areas with stable signals (considering appropriate reporting frequency);

- Upper and lower threshold alarm function;

- Automatic time calibration when connected to the network, automatic reconnection when disconnected;

- Supports trigger operation, and the instrument can be configured on-site;

- Web terminal for viewing and remote configuration of the instrument, supporting data query via mobile phone SMS;

- With shockproof and moisture-proof functions, suitable for field environments;

- Different process connection interfaces can be customized according to customer needs;

- OEM customization of products with different protocols is available.

- Can measure the volume flow and mass flow of steam, gas, and liquid.

- Sensor can be installed and removed without stopping the flow; amplifier and sensor can be separated (separation distance 15m).

- Adopts anti-interference circuit and anti-vibration sensor head, enabling the instrument to have certain resistance to environmental vibration.

- Low pressure loss, wide measuring range, with a range ratio of 10~40 times.

- No mechanical moving parts, long-term stability, simple structure for easy installation and maintenance.

- Measurable medium temperature up to +350℃ (+450℃).

- Modular design, each measurement parameter can be freely combined and flexibly configured;

- Supports the determination of pH, conductivity/TDS, turbidity, residual chlorine/total chlorine, and temperature;

- Supports two measurement modes: continuous measurement and periodic measurement;

- Supports automatic cleaning function;

- Equipped with five electromagnetic relays, supporting external device control;

- Equipped with RS-485 digital interface, supporting data export and remote communication (wired or wireless);

- IP65 protection class.

- Optional color touch screen control panel for on-site data display and control.

- Optional solar panel for outdoor power supply.

Monitoring Center

Market Services

Quality Assurance

Comprehensive Quality Management System Service Guarantee

During the warranty, we are responsible for free repair, replacement, and accuracy verification of measuring devices, and is also responsible for lifetime maintenance.

We are responsible for the training of management personnel free of charge, and the time and place shall be determined by the purchaser. Before installation, we conduct training on on-site installation and commissioning precautions; after installation, conduct training on usage and operation, etc.

During the installation process, we provide on-site guidance services to ensure the normal installation and use of the meters.

Customer First,

Your needs set our roadmap; your feedback drives our next upgrade.

From first hello to lifetime support, every step is shaped around you.

Full Participation

All employees are quality inspectors, participating in the entire process from product design, raw material procurement to product shipment. They identify and report issues to ensure product quality and enable the company to continuously improve product quality throughout this process.

Process Planning

Effectively control and manage the entire process from market research, development and design, production manufacturing to sales and service, ensuring product quality and customer satisfaction.

Process Management

Establishes a full-process quality assurance system covering product research and design, production and manufacturing, after-sales service and other activities. And produce products that satisfy users with the most economical means.

Process Improvement

The PDCA (Plan-Do-Check-Act) cycle-based quality management approach.

请将这里的中国地图改成：世界地图

Quality Service

24/7 hours service hotline      Quick response within 2 hours	   on-site service within 48 hours

Equipped with professional maintenance personnel, 24/7 hours service for customers

Equipped with senior technical personnel to explain the precautions for customers at the user end

Equipped with senior industry marketing personnel to provide you with the highest quality product recommendations and solution formulation

Implement lifetime maintenance for our company's products

Regardless of the cause of product failure, we will reply within 2 hours upon receiving feedback information

Professional Maintenance Team Support

Systematic Technical Training

Solid On-site Service

Lifetime Maintenance

Timely Handling

Professional After-sales Team Covers the Globe